ASpectrum — connections

How the QMTECH Cyclone IV core board, the QM_Atrix adapter, the ESP32 system controller and the microSD card are wired (state of 14 October 2026)

# 1. Overview

Figure 1. What plugs into which header of the core board.

| Part | Connected to | Carries |
| --- | --- | --- |
| QM_Atrix adapter, J2 | U8, plugged in directly (J2.1 on U8.64) | VGA, USB keyboard module, 5 V for the adapter |
| QM_Atrix adapter, J1 | U7, by wires | AY and beeper audio to the amplifier, Kempston joystick, ground |
| ESP32-WROOM-32 DevKit | U7.7-11, U7.17-21 + ground, by wires | SPI link for the tape loader, sound as I2S, UART for the F3 Bluetooth menu |
| Adafruit MicroSD BFF | the ESP32, by wires | SD card (SPI) for the tape loader, 3.3 V |
| Bluetooth headphones / speaker | the ESP32, by radio (A2DP) | the Spectrum's sound |

All FPGA pins are 3.3 V (LVTTL) and not 5 V tolerant. Every signal on the headers is 3.3 V: the adapter makes its own 3.3 V for its pull-ups, the keyboard module's UART is 3.3 V, and the ESP32 is a 3.3 V part.

# 2. The core board headers

U7 is the left header and U8 the right one (32 x 2 pins each). On both: pins 1/2 ground, 3/4 3.3 V, 61/62 ground, 63/64 VIN (5 V). The numbering was read from the QMTECH schematic: check 1/2, 3/4, 61/62 and 63/64 with a meter before connecting anything new. Full header map: docs/BOARD_PINOUT.md.

| On-board | Pin / use |
| --- | --- |
| 50 MHz oscillator | T2 |
| KEY0 (button farther from the long edge) | W13: reset of everything |
| KEY1 | Y13: not used |
| LED | E4: on = turbo; fast blink = no ROMs in the flash (program the .jic) |
| SDRAM, configuration flash | on the board, nothing to wire |

# 3. Adapter J2 on U8

J2 is plugged straight into U8 with J2 pin 1 on U8 pin 64 (so J2 pin k sits on U8 pin 65 - k). J2 pin 2 is cut off on the adapter, because it would sit on VIN. U8 pins not listed are not connected on the adapter.

| Header pin | FPGA pin | Goes to | Signal | Dir | Notes |
| --- | --- | --- | --- | --- | --- |
| U8.64 | - | J2.1 5V0 | 5 V | power | feeds the adapter: its 3.3 V regulator, keyboard module, audio amplifier |
| U8.61, 62 | - | J2.4, J2.3 | GND |  | VGA ground return |
| U8.59 | M20 | BL | VGA_B_LOW | out |  |
| U8.57 | N20 | BH | VGA_B | out |  |
| U8.55 | B22 | GL | VGA_G_LOW | out |  |
| U8.53 | C22 | GH | VGA_G | out |  |
| U8.51 | D22 | RL | VGA_R_LOW | out |  |
| U8.49 | E22 | RH | VGA_R | out |  |
| U8.47 | F22 | HS | VGA_HSYNC | out |  |
| U8.45 | H22 | VS | VGA_VSYNC | out |  |
| U8.19, 17 | AA19, AA18 | keyboard module J5.2 / J5.3 | KBD_A, KBD_B | in | UART 115,200 from the USB keyboard module (both inputs) |
| U8.16, 15 | AB17, AA17 | J2.49, J2.50 = GND | GND_TIE | in | grounded by the adapter: never drive |

U8.1-14 (above the adapter) are not used. U8.5 and U8.6 are clock-only inputs of the FPGA: never use them for anything else.

# 4. Adapter J1, wired to U7

J1 hangs outside the core board; its signals go to U7 by wires. Do not wire J1.1 (5 V) or J1.2 (3.3 V): the adapter is powered through J2.

| Header pin | FPGA pin | Goes to | Signal | Dir | Notes |
| --- | --- | --- | --- | --- | --- |
| U7.1 | - | J1.3 GND | GND |  | ground wire in the bundle |
| U7.15 | J1 | J1.17 AY | AUDIO_AY | out | sigma-delta; 68 Ω + 100 nF filter on the adapter |
| U7.16 | J2 | J1.19 Beeper | AUDIO_BEEPER | out | port FE bit 4 |
| U7.23 | C1 | J1.15 Up | JOY_UP_N | in | 3.3 kΩ pull-ups on the adapter, active low |
| U7.24 | C2 | J1.11 Down | JOY_DOWN_N | in |  |
| U7.25 | B1 | J1.9 Left | JOY_LEFT_N | in |  |
| U7.26 | B2 | J1.7 Right | JOY_RIGHT_N | in |  |
| U7.27 | B3 | J1.13 Fire | JOY_FIRE_N | in |  |

J1.5 (the adapter's 50/60 switch) is not connected: the video mode is chosen with F8.

# 5. ESP32 system controller and microSD card

A 30-pin ESP32-WROOM-32 DevKit (CP2102, USB-C) with the sketch ASpectrum/esp32/esp32.ino. It owns the microSD card, runs the tape loader and the Bluetooth audio, and talks to the FPGA over an SPI link with an interrupt line, an I2S sound input and a UART. The ESP32 drives the link and I2S clocks. ESP32 pins are given by their DOIT 30-pin board labels.

| ESP32 board pin | Signal | Dir | FPGA board pin | FPGA pin | Notes |
| --- | --- | --- | --- | --- | --- |
| D35 | ESP_MISO | FPGA → ESP32 | U7.7 | R1 | link data to the ESP32; D35 is input-only |
| D34 | ESP_IRQ | FPGA → ESP32 | U7.8 | R2 | interrupt, active high; D34 is input-only |
| D14 | ESP_SCK | ESP32 → FPGA | U7.9 | P1 | link clock, 8 MHz; FPGA pull-up |
| D13 | ESP_MOSI | ESP32 → FPGA | U7.10 | P2 | link data to the FPGA; FPGA pull-up |
| D33 | ESP_CS_N | ESP32 → FPGA | U7.11 | N1 | link select; FPGA pull-up (idle while the ESP32 boots) |
| D26 | BT_BCLK | ESP32 → FPGA | U7.17 | H1 | I2S bit clock, 1.4112 MHz; FPGA pull-up |
| D25 | BT_LRCK | ESP32 → FPGA | U7.18 | H2 | I2S word select, 44.1 kHz; FPGA pull-up |
| D22 | BT_DATA | FPGA → ESP32 | U7.19 | F1 | sound, 16-bit; ESP32 pull-down |
| RX2 (GPIO16) | BT_TX | FPGA → ESP32 | U7.20 | F2 | F3 menu commands, 115,200 baud |
| TX2 (GPIO17) | BT_RX | ESP32 → FPGA | U7.21 | E1 | answers and events; FPGA pull-up |
| GND | GND |  | U7.1 / U7.2 | - | required; run it alongside the signal wires |

The microSD card (Adafruit MicroSD BFF) is wired to the ESP32 only. The BFF's jumpers are unchanged (CS = its TX pin); keep 10 µF + 100 nF across 3.3 V and ground at the BFF; its other pins stay unconnected.

| ESP32 board pin | BFF pin | Signal | Notes |
| --- | --- | --- | --- |
| D18 | JP3.2 SCK | SD SCK | ESP32 VSPI, 20 MHz |
| D19 | JP3.3 MISO | SD MISO | ESP32 pull-up; a 10k-47k resistor to 3.3 V is better |
| D23 | JP3.4 MOSI | SD MOSI |  |
| D5 | JP1.7 TX | SD CS |  |
| 3V3 | JP3.5 3.3V | 3.3 V | from the ESP32 board |
| GND | JP3.6 GND | GND |  |

- Power: from the ESP32's own USB-C port (a phone charger is fine), or 5 V from U7.63/64 into the ESP32's VIN pin. Never 5 V on any signal pin.

- Direction check: the FPGA's TX goes to the ESP32's RX2, and the ESP32's TX2 to the FPGA's RX; I2S data and link MISO go from the FPGA to the ESP32.

- Either side unplugged is safe: pull-ups and a pull-down keep the lines quiet; the Spectrum boots and runs without the ESP32 (no SD card, no Bluetooth).

- USB on the ESP32 is for programming and debugging (a terminal at 115,200 baud shows the loader's messages and takes the Bluetooth commands). Pairing is done from ASpectrum: F3 opens the Bluetooth menu.

- Pins avoided on the ESP32 on purpose: GPIO12 (the FPGA's pull-ups would select 1.8 V flash at boot), GPIO2 and GPIO15 (boot strapping), GPIO6-11 (flash).

- Keep the wires short (under ~20 cm). Wiring test: tests/Test_wires/ checks every wire and the SD card and reports on the ESP32's USB port.

# 6. Power

| What | From |
| --- | --- |
| Core board | its own power input (USB or 5 V); everything else on the FPGA side is fed from it |
| QM_Atrix adapter | 5 V from U8.64 through J2.1; the adapter makes its own 3.3 V |
| USB keyboard module | 5 V via the adapter (USB power only; its UART is 3.3 V) |
| microSD BFF | 3.3 V from the ESP32 board |
| ESP32 DevKit | its own USB-C, or 5 V from U7.63/64 into VIN |

# 7. Before switching on

- Meter check of the header power pins (section 2) after any change in the wiring.

- No wire from J1.1 / J1.2 or any 5 V pin to a signal pin; U8.15/16 (grounded by the adapter) are never outputs.

- The configuration flash holds an ASpectrum image (or nothing): a demo image could drive header pins against the adapter.

- Ground wires present on U7 (J1 bundle and ESP32).

- Support the adapter's J1 side with a spacer, so the cables do not lever on U8.