ASpectrum

Architecture of the ZX Spectrum 128K on the QMTECH Cyclone IV board

Design state of 9 October 2026: the 128K machine (on hardware since 6 October), the SD card tape loader with saving, .z80 snapshots and memory contention

1. Overview

2. Block diagram

3. Clocks and resets

4. CPU and clock enable

5. Memory

5.1 SDRAM controller

5.2 Memory map

5.3 Bus bridge

5.4 ROM loading

6. Video

7. Sound, ports and interrupt

7.1 Sound

7.2 I/O ports (partial decoding, as the real 128K)

7.3 Interrupt

8. Keyboard and controls

9. SD card tape loader

9.1 Hardware (tape_loader.v, 56 MHz)

9.2 Pulse player

9.3 Firmware (fw/, C)

9.4 Saving

9.5 Snapshots

10. Pins

11. Timing constraints

12. Verification

13. Known limitations

# 1. Overview

ASpectrum is a ZX Spectrum 128K built in an Intel/Altera Cyclone IV E FPGA (EP4CE15F23C8) on a QMTECH core board. It is a port of the user's earlier Spectrum 128 on a DE10-Lite (MAX10), redesigned around three ideas:

- All memory in SDRAM. The 128 KB of RAM and the ROMs live in the board's 32 MB SDR SDRAM; only the screen is mirrored in block RAM for the video. This keeps the design small enough for smaller FPGAs.

- One system clock, no clock multiplexers. Everything that belongs to the Spectrum runs at 112 MHz; the Z80 is stepped by a clock enable, at the real 3.5469 MHz or at 28 MHz (turbo, 8x).

- Fully constrained timing. Every clock and every I/O has constraints, and the build closes timing at all corners (the DE10 reference had none).

Added on top: an SD card tape loader that plays unmodified .tap and .tzx files in turbo and saves the Spectrum's SAVE output as .tap files, .z80 snapshots (F2 saves the whole machine at any moment, the browser loads them), controlled from on-screen dialogs, and USB-keyboard machine keys (DiagROM, 50/60 Hz, reset).

| Item | Value |
| --- | --- |
| FPGA | EP4CE15F23C8 (15,408 logic elements, 56 M9K block RAMs, 4 PLLs) |
| Board | QMTECH Cyclone IV core board: 50 MHz oscillator, W9825G6KH-6 SDRAM (32 MB, 16 bit), W25Q64 configuration flash, 2 buttons, 1 LED |
| I/O board | User's QM_Atrix adapter: VGA (2 bits per colour), AY + beeper audio to an amplifier module, Kempston DB9, USB keyboard module (UART), tape in, turbo input, 50/60 switch; plus an Adafruit microSD BFF for the tape loader |
| CPU | T80 (Z80, VHDL, from the MiSTer ZX Spectrum core), 3.5469 MHz or 28 MHz |
| Tape loader CPU | PicoRV32 (RISC-V RV32IMC), 56 MHz, 32 KB block RAM, firmware in C |
| Resources used | 9,274 logic elements (60 %), 55 of 56 M9K, 2 PLLs, 71 pins |
| Timing | worst setup slack +0.65 ns (112 MHz, slow 85 °C corner), worst hold +0.11 ns, no negative slack at any corner, no unconstrained paths |
| Tools | Quartus Prime 25.1std Lite, Questa FSE, xPack riscv-none-elf-gcc 15.2 |

# 2. Block diagram

Figure 1. Blocks and clock domains (blue 112 MHz, green 56 MHz, orange pixel clock, grey outside the FPGA).

| Module (rtl/) | Clock | Role |
| --- | --- | --- |
| ASpectrum.v | all | Top level: PLLs, resets, SDRAM clock output, video clock switch, pins |
| zx_system.v | all | The machine without PLLs (what the simulations instantiate): connects all blocks |
| zx_bus.v | 112 MHz | CPU clock enable, bus bridge to SDRAM, paging, I/O ports, interrupt, EAR |
| cpu_t80.v + T80/ | 112 MHz + CEN | Z80 core with a single clock enable |
| sdram_ram.v | 112 MHz | Byte-wide RAM on the SDR SDRAM (open-row, CAS latency 2) |
| rom_loader.v, flash_if.v | 112 MHz | Copies the ROMs from the configuration flash into SDRAM at reset |
| zx_video.v, vga_timing.v | 25 / 27 MHz | Spectrum picture from the screen shadows, borders, FLASH, OSD overlay |
| zx_keyboard.v | 112 MHz | USB keyboard packets to the Spectrum key matrix, loader keys, machine keys |
| jt49/, sd_dac.v | 112 MHz | AY-3-8912 sound chip and its 1-bit audio DAC |
| tape_loader.v + picorv32/ | 56 MHz | SD card tape loader: RISC-V CPU, RAM, SPI, command FIFO, pulse player, recorder (saving), snapshot port |
| sys_pll.v, vga_pll.v, vga_clkmux.v | - | PLLs and the glitch-free pixel clock switch |

# 3. Clocks and resets

| Clock | Source | Used by |
| --- | --- | --- |
| 50 MHz | board oscillator | PLL inputs; video mode switch sequencer |
| 112 MHz (c0) | sys_pll: 50 x 56 / 25 | System: SDRAM controller, CPU + bridge, AY, keyboard, ROM loader |
| 112 MHz delayed (c1) | sys_pll, +1.34 ns | SDRAM clock, forwarded inverted through a DDIO output |
| 56 MHz (c2) | sys_pll: 50 x 28 / 25 | Tape loader; rising edges aligned with c0, so crossings are ordinary timed paths |
| 25 / 27 MHz | vga_pll | Pixel clock for 640x480@60 or 720x576@50, one at a time through the global clock control block |

Video clock switch. Changing between 50 and 60 Hz is a sequence run from the 50 MHz clock: video reset on, clock disabled, select changed, clock enabled, video reset off. The clock control block makes the switch glitch-free.

Resets. Three reset domains, each released synchronously:

- Power-up (PLL locked): the keyboard decoder. Held keys and the 50/60 choice therefore survive a machine reset, which is what makes "hold F1 and reset" work.

- Machine (power-up, KEY0, Ctrl+Alt+Del): SDRAM controller, ROM loader, CPU, ports, AY.

- Tape loader (power-up, KEY0): the RISC-V subsystem. Ctrl+Alt+Del leaves it running.

# 4. CPU and clock enable

The Z80 is the T80 core, copied unchanged from the MiSTer ZX Spectrum core, wrapped in cpu_t80.v with a single clock enable (CEN). All T80 state changes only on CEN, so the CPU can be slowed, sped up or stopped at any clock without corrupting anything.

- CEN generator: a 32-bit phase accumulator at 112 MHz. Normal speed adds 136,016,246 per clock, giving exactly 3.5469 MHz on average (31-32 clocks per T-state, the 128K's frequency). Turbo adds 2^30, giving exactly 28 MHz (one T-state every 4 clocks). Switching is glitch-free at any time.

- Turbo sources: the TURBO_N pin (external tape simulator, active low) or the SD tape loader while it plays or saves a block; F6 switches the loader to normal speed.

- Hold: the tape loader can stop the CEN (like a memory wait) while it needs time during a save; the Spectrum does not notice, because its T-states stop too.

- Snapshot freeze: the tape loader can stop the CEN at an instruction boundary (section 9.5).

- Memory contention (Level 1, 128K timing): a frame T-state counter restarts at each interrupt and counts real T-states. At the start of each memory cycle to 4000-7FFF (or to C000-FFFF while an odd RAM page is paged in), and in the I/O patterns of the 128K, the CEN is withheld by 6, 5, 4, 3, 2, 1, 0, 0 T-states during the 128 contended T-states of each of the 192 picture lines (from T-state 14,361). Those T-states are lost, as on the real machine, and the tape player counts them too. Off in turbo; F5 switches it off. Not emulated: the internal contended T-states of some instructions.

- Floating bus: an I/O read of a port nothing answers (e.g. port FF) returns the byte the ULA is fetching when the CPU reads the port, as on a real 128K. The read point follows Fuse 1.6: the start of the I/O cycle's 4th T-state, after all of its contention. Counting x = T-state - 14,362 - 228 x row, the bitmap and attribute bytes of the displayed screen appear at x = 2-5 of each 8 within the 128 picture T-states of a line, FF elsewhere. A contended poll (port high byte 40-7F) therefore only ever sees an attribute or FF, which games that sync on an attribute rely on (Sidewize). The bridge decides at the start of T2: it takes hpos and adds the contention the cycle is still going to get (two pipeline stages), then reads the byte from SDRAM before the CPU takes it at the end of the wait state. (Until 11 October the byte was taken at T2; half of Sidewize's polls then saw pixel bytes, the game synced too early and its player flickered.) On together with contention (F5), off in turbo. Not emulated: ULA snow (DiagROM's snow test fails).

- Stall: if a memory read is not finished when the CPU needs its data, the CEN pulse is held back ("owed") and given as soon as the data is there. At normal speed this never happens (the SDRAM is always fast enough); in turbo it costs a clock now and then.

- Timing constraints: CEN pulses are never closer than 4 clocks, so paths inside the T80 get a 4-clock multicycle (the T80 alone reaches 63.5 MHz on this chip, so 4 x 8.9 ns is ample).

# 5. Memory

## 5.1 SDRAM controller

sdram_ram.v turns the 16-bit W9825G6KH-6 (4 banks x 8192 rows x 512 columns) into a simple byte-wide RAM with a request/acknowledge handshake. It runs at 112 MHz with CAS latency 2 and was proven on its own in the DDR_TEST project before being used here.

- Address mapping: bit 0 selects the byte lane (DQM), bits 9-1 the column, bits 11-10 the bank, higher bits the row. Consecutive 1 KB blocks rotate through the 4 banks.

- Open-row policy: each bank keeps its last row open, so code and data that stay within a few kilobytes are read without an ACTIVATE.

- Refresh: done early whenever the controller is idle, forced only when the 7.5 µs interval runs out.

- Pins: all SDRAM address, command and data pins use I/O-cell registers; the SDRAM clock is the inverted, slightly delayed system clock, forwarded through a DDIO output so it has the same output delay as the data.

| Access | Clocks from request to acknowledge |
| --- | --- |
| Write | 1 (posted: the controller completes it on its own) |
| Read, row already open | 4 |
| Read, bank idle | 6 |
| Read, other row open in that bank | 8 |
| Read colliding with a refresh (worst case) | about 20 |

## 5.2 Memory map

The Spectrum uses an 18-bit (256 KB) window of the SDRAM:

| SDRAM address | Contents |
| --- | --- |
| 0x00000 - 0x1FFFF | RAM pages 0-7, 16 KB each (page n at n x 0x4000) |
| 0x20000 - 0x23FFF | ROM 0: 128K editor |
| 0x24000 - 0x27FFF | ROM 1: 48K BASIC |
| 0x28000 - 0x2BFFF | DiagROM v1.59 (used for both ROM slots when selected at start) |
| 0x2C000 - 0x3FFFF | spare |

| CPU address | Mapped to |
| --- | --- |
| 0x0000 - 0x3FFF | ROM 0 or ROM 1 (port 7FFD bit 4), or DiagROM; writes are ignored |
| 0x4000 - 0x7FFF | RAM page 5 (screen 0) |
| 0x8000 - 0xBFFF | RAM page 2 |
| 0xC000 - 0xFFFF | RAM page 0-7 (port 7FFD bits 2-0); page 7 holds screen 1 |

## 5.3 Bus bridge

zx_bus.v connects the T80 to the SDRAM. It watches the CPU's machine cycles and their T-states:

- T1, first clock: the address is latched and a read is started at once (speculatively), mapped through the paging logic. The SDRAM therefore has almost two T-states before the CPU takes the data.

- T1, second clock: the cycle type (memory or I/O, read or write) is sampled.

- T2: for writes, the data is sampled and the write is posted: the CPU does not wait for it. The next read waits until earlier writes are done, so a read always sees them. Writes to the screen areas of pages 5 and 7 also go to the screen shadows.

- End of T2: the CEN that lets the CPU take the data is held until the read has completed (only possible in turbo).

## 5.4 ROM loading

The ROMs are stored in the configuration flash at 0x100000 (128K set, 32 KB) and 0x108000 (DiagROM, 16 KB), written together with the bitstream as one .jic file. After every machine reset rom_loader.v reads the 48 KB with SPI READ commands through the Cyclone IV ASMI block and writes them into SDRAM (about 14 ms), then the CPU starts. The .jic tool stores user data bit-reversed; the loader recognises this from the first byte (F3 = DI, or CF = F3 reversed) and corrects it. A missing ROM image makes the LED blink fast.

At power-up the CPU additionally waits for the first USB keyboard packet (at most 1.5 s), so that a held F1 key is already known when the ROM is chosen.

# 6. Video

The picture is generated in its own pixel clock domain from screen shadows: block-RAM copies of the first 6912 bytes (bitmap + attributes) of RAM pages 5 and 7, written in parallel with the CPU's SDRAM writes. The video never reads the SDRAM, so it never disturbs the CPU.

| Mode | Timing | Pixel clock | Picture placement |
| --- | --- | --- | --- |
| 50 Hz | 720x576 (CEA 576p), 864 x 625 | 27.000 MHz | 512x384 picture, 104 px / 96 lines border |
| 60 Hz | 640x480, 800 x 525 | 25.000 MHz | 512x384 picture, 64 px / 48 lines border |

- Each Spectrum pixel is 2 x 2 VGA pixels. Bitmap and attribute bytes are fetched 4 pixels ahead; FLASH swaps ink and paper every 16 frames; BRIGHT drives the low DAC bit.

- Power-up mode is 60 Hz (640x480, accepted by every monitor; 576p50 is not); F8 swaps it, and the choice is kept across resets. The adapter's 50/60 switch is not read.

- OSD: 32 x 24 characters exactly over the Spectrum picture, using the Spectrum's own character set (taken from the 48K ROM), white on blue; bit 7 of a character means inverse video. It can cover the whole picture (browser) or only the bottom row (status). It uses the same 4-pixel-ahead fetch: text RAM, then font ROM.

# 7. Sound, ports and interrupt

## 7.1 Sound

- AY-3-8912: JT49 core (GPL-3) clocked at 1.7734 MHz as on the 128K, independent of turbo. The three channels are mixed into one 10-bit value and output by a sigma-delta DAC at 112 MHz on AUDIO_AY; the adapter's 68 Ω + 100 nF filter makes it analog again.

- Beeper: port FE bit 4, output directly on AUDIO_BEEPER.

## 7.2 I/O ports (partial decoding, as the real 128K)

| Port | Function |
| --- | --- |
| Write, A0 = 0 (FE) | border colour (bits 2-0), MIC (bit 3, to the tape loader's recorder; no pin), beeper (bit 4) |
| Write, A15 = 0, A1 = 0 (7FFD) | RAM page (2-0), screen page 5/7 (3), ROM (4), lock until reset (5) |
| Write FFFD / BFFD | AY register select / data |
| Read, A0 = 0 (FE) | keyboard half-rows selected by A8-A15 (bits 4-0), EAR (bit 6), bits 5 and 7 = 1 |
| Read FFFD | AY register |
| Read, A0 = 1, A5 = 0 (1F) | Kempston joystick: 000FUDLR, 1 = pressed |
| Other reads | floating bus: the screen byte the ULA is fetching (section 4), FF in the border, in turbo or with F5 off |

EAR (bit 6) comes from, in order of priority: the SD tape loader while it plays; the TAPE_IN pin for 75 ms after each edge on it; otherwise the beeper bit (as on the real machine).

## 7.3 Interrupt

The 32-T-state interrupt comes from the video in 50 Hz mode (a true 50.00 Hz frame) and from a 70,908-T-state counter (the 128K frame) in 60 Hz mode; the counter follows turbo.

- In 50 Hz mode the interrupt fires at a set point of the VGA frame, so that border effects line up with the picture: VGA line 24, pixel 166 after the start of vsync, measured with Aquaplane's horizon stripe. Page Up / Page Down (browser closed) move it in 1/8-line steps. Calculated for a real 128K it would be line 12.7; the difference is most likely the contention that is not emulated (Level 2).

- In 60 Hz mode the picture is not locked to the Spectrum frame (59.5 Hz video, 50 Hz interrupts), so border effects cannot line up.

# 8. Keyboard and controls

A USB keyboard is attached through a CH9350-style module that sends HID reports over a 115,200-baud UART. zx_keyboard.v decodes the modifier byte and the first three key codes of each report into the 8 x 5 Spectrum key matrix, plus extras:

- PC keys mapped to Spectrum combinations: Backspace = DELETE, arrows = cursor keys, Esc = BREAK, Left Shift = CAPS SHIFT, Right Shift / Ctrl = SYMBOL SHIFT.

- Machine keys handled in hardware: F1 held at start (DiagROM), F8 (50/60 Hz), Ctrl+Alt+Del (machine reset).

- Loader keys passed to the tape loader CPU as a bit vector: F12 / numpad keys / arrows / Enter / Esc / F2 (snapshot) / F5 (contention) / F6 (speed) / F7 (stop) / F9-F11 / Page Up / Page Down (browser paging, or the 50 Hz interrupt position); plus the raw keyboard report for typing file names.

- While the browser is open the Spectrum's key matrix is released, so it sees no keys.

The complete key list with usage cases is in the separate document ASpectrum_Keys.docx. Board buttons: KEY0 resets everything; KEY1 is not used.

# 9. SD card tape loader

The loader replaces the user's external tape simulator (a small microcontroller playing pre-converted files). It reads unmodified .tap and .tzx files from a FAT16/FAT32 microSD card and feeds them to the Spectrum's EAR input in turbo, and it records the Spectrum's SAVE output into new .tap files on the card.

## 9.1 Hardware (tape_loader.v, 56 MHz)

- PicoRV32 (RISC-V RV32IMC, ISC licence, unmodified source) without interrupts or counters, with compressed instructions and multiply/divide (smaller code); 2,580 logic elements, register file in 2 M9K.

- 32 KB RAM for code, data and stack, built as four 8-bit block RAMs (one per byte lane) and loaded with the firmware as part of the bitstream.

- Recorder: MIC edges timed in T-states; while a save runs, an unread edge holds the Z80, so nothing is lost.

- SPI master for the card: 400 kHz while initialising, 14 MHz afterwards.

- Command FIFO (512 x 32 bit) and the pulse player.

- OSD write port into the text RAM of the video block; timer; control register for turbo, EAR source and OSD.

## 9.2 Pulse player

Every tape block is a sequence of signal edges. The CPU turns the file into commands; the player executes them, counting time in Z80 T-states (it counts the CPU's clock-enable pulses), not in nanoseconds. Consequences: the same file is exact at 3.5 MHz and at 28 MHz, SDRAM stalls stretch tape and CPU equally, and custom turbo loaders stay in step with the Z80.

| Command | Meaning |
| --- | --- |
| PULSE n | hold the level for n T-states, then toggle |
| LEVEL l, n | set the level, hold it n T-states (n = 0: no time) |
| DATA b, k | k bits of byte b, most significant first; each bit = 2 pulses of length LEN0 or LEN1 |
| SAMPLES b, k | k bits of b; each sets the level for LEN0 T-states (TZX direct recording) |
| LEN0, LEN1, MARK | settings: bit lengths, block number (no time) |

The next command starts in the same clock as the previous one ends, so there are no gaps; settings are taken in while a pulse runs and take effect exactly at its end, so they never show on the line. If the CPU is late, the delay is credited to the next pulse (up to 255 T-states), so edges stay on time.

## 9.3 Firmware (fw/, C)

- SD driver: SPI-mode initialisation and single-block reads for SD, SDHC and SDXC cards.

- FAT16/FAT32: with or without a partition table, long file names, fragmented files, seeking.

- TAP and TZX: TZX blocks 10, 11, 12, 13, 14, 15, 20 and 2B are played; 21-27 (groups, jumps, loops, calls) are followed; 18, 19, 28, 2A and information blocks are skipped.

- Browser and control: sorted folder lists (up to 350 entries) on the OSD, keys with auto-repeat, pause, back one block, stop, turbo/normal speed (F6), and "stop the tape" blocks for multi-load games.

- Saving: recording mode, ROM-format decoder (pilot, sync, bits), 8.3 name typed on the USB keyboard, FAT writing (free clusters, both FAT copies, directory entries, growing folders, deleting an empty file), SD block writes.

- Snapshots: .z80 version 3 writer (pages compressed in two passes: length, then data) and a loader for versions 1-3, 48K and 128K, that checks the whole file before changing anything.

The firmware is 15.7 KB of code plus 15.3 KB of data (built with shared prologue/epilogue code, -msave-restore); all variables are cleared at start, because a reset does not reload the RAM image. The browser limit went from 400 to 350 entries to make room for the snapshot code.

## 9.4 Saving

Saving is a recording mode, started from the browser (first line [Save to this folder], then an 8.3 name) and stopped with F12. Meanwhile every ROM-format block the Spectrum saves is decoded from MIC and appended to that one .tap file; a block needs 64 pilot pulses, so MIC clicks for sound are ignored, and turbo is switched on only while a block is being saved. TAP block lengths are written as 0 first and patched when a block ends. Each bit is decided by its first pulse, because the ROM does not always end a block with an edge. An empty recording removes its file.

## 9.5 Snapshots

F2 saves the whole machine as a .z80 file (version 3, 128K): the 8 RAM pages, every CPU register, port 7FFD, border, the AY registers and its register select. Choosing a .z80 file in the browser loads it (versions 1-3; 48K snapshots run in 48K mode with 7FFD = 30h).

- Freeze at an instruction boundary (zx_bus): the CEN stops in T2 of an opcode fetch that is not an interrupt acknowledge and does not follow a CB/ED/DD/FD prefix (tracked from the fetched opcodes). There the T80 has written back the previous instruction (at T1) and not yet taken the opcode or incremented PC and R, so its registers are the state between two instructions. In HALT the PC is already past the HALT: the firmware saves PC - 1.

- Registers are read from the T80's REG output; loading resets the T80 alone and loads every register at once through its DIRSet/DIR port (the MiSTer way), then the CPU starts a fresh opcode fetch at the saved PC.

- Command port (loader to zx_bus, request toggle + acknowledge): SDRAM byte read / write (writes also update the screen shadows), register word read, register word write + LOAD, AY register read / write / select, ports (7FFD, border), state (HALT, AY select, ports).

- Unfreeze: the opcode read of the stopped M1 is repeated (the loader's SDRAM reads changed the data bus), so the program continues as if nothing happened; T-states stopped too, so a playing tape stays in step.

- Not saved: the position inside the video frame; after a load the next interrupt comes at a different point of the frame (the T80's reset state also costs one T-state).

# 10. Pins

| Signal | FPGA pin | Where |
| --- | --- | --- |
| CLOCK_50 | T2 | board oscillator |
| RESET_N (KEY0) | W13 | board button |
| LEDR | E4 | board LED: on = turbo, fast blink = no ROMs in flash |
| DRAM_* (40 pins) | various | on-board SDRAM (as the QMTECH demo) |
| VGA_R/G/B, _LOW | E22 D22 C22 B22 N20 M20 | adapter J2 on U8: 2-bit DAC per colour |
| VGA_HSYNC / VSYNC | F22 / H22 | adapter J2 on U8 |
| TAPE_IN / TURBO_N | Y22 / AA20 | adapter J2: external tape simulator |
| KBD_A / KBD_B | AA19 / AA18 | adapter J2: keyboard module UART (both inputs, data = A AND B) |
| GND_TIE[1:0] | AA17 / AB17 | tied to ground by the adapter: inputs only |
| AUDIO_AY / AUDIO_BEEPER | J1 / J2 | adapter J1 wired to U7.15 / U7.16 |
| JOY_UP/DOWN/LEFT/RIGHT/FIRE_N | C1 C2 B1 B2 B3 | adapter J1 wired to U7.23-27 (pull-ups) |
| SD_MOSI / MISO / SCK / CS_N | AA13 / AA14 / AA15 / AA16 | Adafruit microSD BFF on U8.7 / 9 / 11 / 13 |

All pins are 3.3-V LVTTL; unused pins are inputs. The full header map is in docs/BOARD_PINOUT.md.

# 11. Timing constraints

- All PLL clocks derived; the 50 MHz, system (112 / 56 MHz and the SDRAM clock) and pixel clock groups are asynchronous to each other, and every crossing between them is synchronised or goes through a dual-clock block RAM. 112 and 56 MHz come from one PLL with aligned edges and are timed against each other normally.

- SDRAM pins: input and output delays from the W9825G6KH-6 data sheet plus board skew; read data has a 2-clock setup multicycle from the SDRAM clock.

- T80 internal paths: 4-clock multicycle; cycle decode sampled by the bridge: 2-clock multicycle.

- SDRAM owner switch (ROM loader to CPU): 2-clock multicycle, valid because both sides are idle when it happens once after reset.

- Board inputs and slow outputs (buttons, switches, keyboard, joystick, audio, VGA DAC, SD card SPI) are false paths; the SPI protocol itself leaves several clocks of margin.

# 12. Verification

| Test | What it shows |
| --- | --- |
| sim/run_sim.sh | Whole machine with the real ROMs: flash model, ROM loader, SDRAM model with timing checks, T80 boots the 128K ROM to its menu (and DiagROM with F1) |
| sim/run_loader_sim.sh | Tape loader with an SD card model: firmware boots, browser shown, a TZX with every block type is played T-state exact against the reference; stop block, continue, Stop key; recording started from the browser, a ROM-style save with the CPU hold, F12, file checked on the card image |
| sim/run_tapeload_sim.sh | Whole machine: the 128K "Tape Loader" loads and runs a BASIC program from the SD card image through the player (result pending at the time of writing) |
| fw/test/run_host_test.sh | Firmware on the PC against generated FAT16/FAT32 card images (fragmented files, long names): every listing and every TAP/TZX signal compared with an independent Python reference; recordings of real TAP files (noise, headerless, several saves, 1 s pauses in turbo, empty) written and checked by an independent FAT checker; snapshots: reference .z80 files (versions 1-3, 48K/128K, stored pages, bad files) loaded into a model of the hardware, and save + reload round trips checked by an independent .z80 decoder |
| sim/run_snap_sim.sh | zx_bus + T80 + SDRAM: a Z80 test program (prefixes, DD CB, block instructions, EXX, IM 2, HALT, paging, AY) run undisturbed, with random freezes and reads, and with random freezes and full restores (LOAD): identical results |
| sim/run_cont_sim.sh | zx_bus + T80 + SDRAM: 8,000 NOPs in contended RAM run 57 per line in the border and 41 per picture line (the known Spectrum result), 57 everywhere with contention off |
| sim/run_float_sim.sh | zx_bus + T80 + SDRAM: 6,000 IN A,(FF) against Fuse 1.6's rule for the 128K floating bus (read point after all I/O contention; incl. contended port addresses); all FF with it switched off |
| sim/run_floatgame_sim.sh | zx_bus + T80 + SDRAM running a real game from a .z80 (default Sidewize): every floating-bus read and every exit of the game's sync loop compared with Fuse's rule (~1 minute per frame) |
| sim/run_snapsys_sim.sh | Whole machine with the firmware: F2, a typed name, the snapshot written to the SD card model, F12 + Enter loads it back; loaded state equals the saved state; the file is checked by the independent decoder |
| DDR_TEST, VGA_TEST | Earlier stand-alone projects that proved the SDRAM controller and the video modes on the hardware |
| Hardware | 128K boots and runs (6 October); games load from the SD card in turbo and at normal speed, saving to the card works (8 October); 60 Hz start, contention and the 50 Hz border alignment confirmed with Aquaplane, floating bus confirmed with Sidewize, snapshots save and load (9 October); floating bus read point corrected, Sidewize's player no longer flickers, Arkanoid and Cobra work (11 October) |

# 13. Known limitations

- Memory contention only at Level 1 (memory and I/O cycles; not the internal T-states some instructions contend); no ULA snow: timing-critical demos are not exact. The 50 Hz interrupt position is tuned (Aquaplane) rather than calculated; other border-effect games may need Page Up / Page Down.

- The AY output is mono; MIC is not on a pin (saving goes to the SD card only).

- Tape loader: TZX generalized data (0x19), CSW (0x18) and the select-block menu (0x28) are not played; text and message blocks are not shown; LOAD "" is not typed automatically; at most 350 entries per folder; exFAT cards are not supported.

- Saving: ROM format only, 8.3 names, no overwrite, no file dates. Snapshots: .z80 only (no .sna), no overwrite, the video frame position is not kept.

- Block RAM: 55 of 56 M9K are used.