ASpectrum — extra keys

ZX Spectrum 128K on the QMTECH Cyclone IV board, with the SD card tape loader, snapshots and memory contention (build of 9 October 2026)

All keys below are on the USB keyboard unless a board button is named. Keys that the Spectrum itself uses (letters, digits, Enter, Space, Shift) work as on a real 128K and are listed at the end.

# Machine keys

| Key | What it does | Notes |
| --- | --- | --- |
| F1 (held while the Spectrum starts) | Starts DiagROM (test ROM) instead of the 128K ROM | Only read at the moment the Spectrum starts: at power-up, after KEY0, or after Ctrl+Alt+Del. Holding F1 later does nothing. Keep holding F1 until the DiagROM screen appears. See "Starting the test ROM" below. |
| F2 | Saves a snapshot of the whole machine as a .z80 file | The Spectrum stops at once and asks for a name. See "Saving and loading snapshots" below. |
| F5 | Switches memory contention on (default) / off | Contention is the slowing of the processor while the screen is drawn, as on a real Spectrum; timing-critical games and border effects need it. F5 also switches the floating bus (port FF reads the screen, as on the real machine; a few games use it, e.g. Sidewize, Arkanoid and Cobra, and stop with F5 off). Shows "Contention: on" or "off" for 4 seconds. Back to on after KEY0 or power-up. Never active in turbo. |
| F6 | Switches tape loading and saving between turbo (default) and normal speed | Shows "Speed: normal" or "Speed: turbo" on the bottom row for 2 seconds. Works at any time; a tape that is playing or a block being saved changes speed at once. Back to turbo after KEY0 or power-up. |
| F8 | Swaps the video between 50 Hz (720x576) and 60 Hz (640x480) | Each press swaps. The choice survives resets. At power-up the video is always 60 Hz (640x480), which every monitor accepts; not all monitors accept the 50 Hz mode. The 50/60 switch on the adapter is not used. Border effects (stripes in the border that line up with the picture) only work in the 50 Hz mode. |
| Page Up / Page Down (browser closed) | Moves the frame interrupt 1/8 line earlier / later (50 Hz mode) | Lines up border effects with the picture: Page Down moves them down. Shows "Frame INT: 24.1" (line.eighth) for 4 seconds; 24.1 is the default, measured with Aquaplane. Back to 24.1 after KEY0 or power-up. With the browser open these keys page through the list. |
| Ctrl + Alt + Del | Resets the Spectrum | Like pressing reset on the machine: the ROMs are reloaded and the 128K menu comes back. The SD tape loader is not reset: a tape that is playing keeps its place (pause it first with keypad 5 if you want it to stop). |
| KEY0 (board button W13) | Resets everything, including the SD tape loader | The video mode is kept. |
| KEY1 (board button Y13) | Not used | It used to select DiagROM and swap 50/60; that is now F1 and F8. |

# SD tape loader keys

The loader shows a file browser over the Spectrum picture (white on blue). Every action has a numpad key and an F-key, so keyboards without a numpad work too. While the browser is open the Spectrum sees no keys at all.

| Key | Browser closed | Browser open |
| --- | --- | --- |
| F12, keypad / or NumLock | Opens the browser; while recording: stops recording | Closes the browser |
| Keypad 8 / 2, F9 / F10, Up / Down | (Up / Down go to the Spectrum as cursor keys) | Moves the selection up / down (held: repeats) |
| Keypad 4 / 6, Left / Right, Page Up / Page Down | (Left / Right go to the Spectrum) | One page (20 lines) up / down; held: repeats |
| Enter, keypad Enter | (goes to the Spectrum) | Plays the selected tape, loads the selected .z80 snapshot, or opens the folder |
| F11 | Pauses / continues the tape | Same as Enter |
| Keypad 5 | Pauses / continues the tape | Pauses / continues the tape |
| Keypad - | Goes back one block on the tape | Goes back one block on the tape |
| F7, keypad * | Stops the tape: normal speed and normal EAR again, nothing shown on screen | Stops the tape |
| Esc, Backspace | (Esc = BREAK, Backspace = DELETE on the Spectrum) | Goes up one folder; in the top folder closes the browser |
| F2 | Saves a snapshot into the folder the browser was last in | Saves a snapshot into this folder, then closes the browser |

The browser stays on screen until you let go of the key that closed it, so the Spectrum never sees that Enter or Esc (Esc would be BREAK).

## Name keys

After choosing [Save to this folder] in the browser, or after F2:

| Key | Action |
| --- | --- |
| Letters, digits, -, _ | Type the name (up to 8 characters; .TAP or .Z80 is added) |
| Backspace | Deletes the last character |
| Enter | Starts recording / saves the snapshot |
| Esc | Back to the browser / no snapshot, the Spectrum runs on |

While recording, F12 (or F7) stops recording and closes the file; the browser cannot be opened then.

# Usage cases

## Loading a game

- Make the Spectrum wait for a tape: choose Tape Loader in the 128K menu, or type LOAD "" and press Enter.

- Press F12. The browser shows the folders and the .tap / .tzx files of the SD card.

- Pick the file (arrows, keypad 8/2) and press Enter. The browser closes and the tape starts.

- While the tape plays, the Spectrum runs at 28 MHz (8 times faster) and the board LED is on. At the end of the tape it goes back to normal speed by itself. Press F6 before (or during) loading to load at the normal speed instead.

## Multi-load games and "stop the tape" blocks

- Some TZX files stop the tape between parts. The bottom row then shows Tape stopped (5=go on) and the speed returns to normal.

- When the game asks for the next part, press keypad 5 (or F11). Loading continues where it stopped.

## A loading error, or starting a block again

- Press keypad 5 to pause the tape. The bottom row shows Paused, block N.

- Press keypad - once for every block you want to go back.

- Make the Spectrum wait for the tape again (for example LOAD ""), then press keypad 5 to play from that block.

## Saving programs

- Press F12, go into the folder you want, and choose the first line [Save to this folder].

- Type a name, or keep the suggested SAVEnnnn, and press Enter. The browser closes and recording starts; the bottom row shows Rec NAME.TAP: 0 F12=stop.

- Save on the Spectrum as often as you like, for example SAVE "game", SAVE "pic" SCREEN$ or SAVE "code" CODE 32768,1000. Every block goes into this one file. Blocks are saved at 8 times the speed; in between the Spectrum runs normally.

- Press F12 to stop. The bottom row shows Saved NAME.TAP (n blocks). If nothing was saved, the empty file is removed.

To load it back: F12, choose the file, Enter (after LOAD "" as usual). Only the ROM's standard save format is recognised.

## Saving and loading snapshots

A snapshot is the whole machine at one moment: memory, processor, sound chip, border. Load it later and the program goes on from exactly there, for example a game at level 5.

- At the moment you want to keep, press F2. The Spectrum stops at once (the sound goes quiet) and asks for a name.

- Type a name (up to 8 letters or digits) and press Enter. The file NAME.Z80 is written into the folder the browser was last in (the top folder after power-up; to save elsewhere, open the browser with F12, go into the folder, then press F2 there).

- After about a second the Spectrum runs on exactly where it stopped, and the bottom row shows Saved NAME.Z80. Esc instead of Enter saves nothing.

- Press F12, choose the .z80 file and press Enter. No LOAD "" is needed: the program starts at once from the saved moment.

- Snapshots from emulators work too: .z80 files of 48K and 128K programs. 48K programs run in 48K mode, as on a real 128.

A name that already exists is refused: choose another. F2 does nothing while the tape recorder is recording (press F12 first). Loading a snapshot stops a tape that is playing.

## Border effects in the wrong place

Some games draw stripes in the border at a fixed moment, for example the horizon in Aquaplane. They only line up with the picture in the 50 Hz mode.

- Press F8 once for the 50 Hz mode (720x576), then load the game.

- If the border stripe sits above or below where the game draws it, press Page Down (stripe moves down) or Page Up (up), with the browser closed, until it lines up. Each press moves it 1/8 line; the bottom row shows the position.

- Note the number shown. The setting goes back to 24.1 after KEY0 or power-up.

## Starting the test ROM (DiagROM)

- Press and hold F1.

- While holding F1: switch the board on, or press KEY0, or press Ctrl + Alt + Del.

- Keep holding F1 until the DiagROM screen appears, then let go.

- To get back to the normal 128K ROM, reset without holding F1 (Ctrl + Alt + Del or KEY0).

At power-up the Spectrum waits up to 1.5 seconds for the keyboard so that a held F1 is seen. If DiagROM still does not start, keep holding F1 and press Ctrl + Alt + Del: that always works.

## Stopping a tape completely

Press F7 (or keypad *) at any time. The tape stops, the Spectrum goes back to normal speed, the status row disappears and the tape position is forgotten. To play the file again, choose it in the browser (F12): it starts from the beginning. Pause (keypad 5) is different: it keeps the position and shows a status row.

## Resetting

- Ctrl + Alt + Del: the Spectrum restarts; a tape that is playing is not touched.

- KEY0: everything restarts, including the tape loader (the SD card is read again).

# Good to know

- SD card: FAT32 or FAT16. exFAT does not work (most cards larger than 32 GB come as exFAT: format them as FAT32).

- Folders work. The browser shows folders first, then files, sorted by name, up to 350 per folder; long names are shown up to 27 characters.

- Unmodified .tap and .tzx files are played and .z80 snapshots are loaded; no conversion on the PC is needed.

- The external tape simulator on the adapter (TAPE_IN, TURBO_N) still works as before.

- If the browser says No SD card, or not FAT16/FAT32, insert the card and press Enter to try again.

# Spectrum keys on the USB keyboard

| USB key | Spectrum key |
| --- | --- |
| Letters, digits, Enter, Space | The same keys |
| Left Shift | CAPS SHIFT |
| Right Shift or Ctrl | SYMBOL SHIFT |
| Backspace | DELETE (CAPS SHIFT + 0) |
| Arrow keys | Cursor keys (CAPS SHIFT + 5 / 6 / 7 / 8) |
| Esc | BREAK (CAPS SHIFT + SPACE) |